- CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

| Product NameProd Name | Cellulose Acetate Butyrate (CAB-381-2) |
| --- | --- |
| Product Identification Number(s)PM/SPC/ZPPID | 01874-00, P01874AB, P0187401, P0187402, P0187403, P0187407, P0187432, P0187452, P0187400, P0187433, E0187401, P01874SB |
| Manufacturer/SupplierManuf/Suppl | Eastman Chemical Singapore Pte Ltd (ECSPL) |
| MSDS Prepared byMSDS Prepared by | Eastman Product Safety and Stewardship |
| Chemical NameChem Name | not applicable |
| Synonym(s)Synonyms | 321881 |
| Molecular FormulaMolec Form | not applicable |
| Molecular WeightMolec Weight | not applicable |
| Product UseProd Use | polymer |
| OSHA StatusOSHA Status | nonhazardous |

For emergency health, safety and environmental information, telephone (60) 9-583-9696 in Malaysia or (65) 738-4877 in Singapore or 423-229-2000 in USA.

For emergency transportation information, telephone England: 0800 964214 or Eire: 01800 559700 (24-hour toll free), or for countries other than England or Eire telephone 1 423 229 2000 (USA) and identify the call as a transportation emergency.

- COMPOSITION INFORMATION ON INGREDIENTS

(Typical composition is given, and it may vary.  A certificate of analysis can be provided.)

| Weight % | Component | CAS Registry No. |
| --- | --- | --- |

| Average Value100Lower ValuePuts Dash in if necessaryUpper ValueUOM% | cellulose acetate butyrate | 9004-36-8 |
| --- | --- | --- |

- HAZARDS IDENTIFICATION

CAUTION!

POWDERED MATERIAL MAY FORM EXPLOSIVE DUST-AIR MIXTURES

| HMIS® Hazard Ratings:HMIS | Health - 0, Flammability -1, Chemical Reactivity - 0 |
| --- | --- |

HMIS® rating involves data interpretations that may vary from company to company. They are intended only for rapid, general identification of the magnitude of the specific hazard. To deal adequately with the safe handling of this material, all the information contained in this MSDS must be considered.

- FIRST-AID MEASURES

Inhalation

Inhalation: If symptomatic, move to fresh air. Get medical attention if symptoms persist.

Eyes

Eyes: Any material that contacts the eye should be washed out immediately with water. If easy to do, remove contact lenses. Get medical attention if symptoms persist.

Skin

Skin: Wash with soap and water. Get medical attention if symptoms occur.

Ingestion

Ingestion: Material is not expected to be absorbed from the gastrointestinal tract so that induction of vomiting should not be necessary.

Notes to Phys

- FIRE FIGHTING MEASURES

Ext Media Use…Extinguishing Media:  water spray, dry chemical

Special FireFighting Proc

Special Fire-Fighting Procedures: Wear self-contained breathing apparatus and protective clothing.

Haz Combustion Products

Hazardous Combustion Products: carbon dioxide, carbon monoxide

Unusual Fire and Explosion Hazards

Unusual Fire and Explosion Hazards: Powdered material may form explosive dust-air mixtures.

Sensitivity to static Discharge

Sensitivity to Static Discharge: Material may accumulate a static charge which could act as an ignition source.

- ACCIDENTAL RELEASE MEASURES

Sweep or scoop up and remove.

- HANDLING AND STORAGE

Personal Precautionary Measures

Personal Precautionary Measures:  No special precautionary health measures should be needed under anticipated conditions of use.

Prevention of Fire and Explosion

Prevention of Fire and Explosion: Keep from contact with oxidizing materials.  Mixing cellulose esters in a nonpolar hydrocarbon, such as toluene or xylene, may result in the buildup of static electricity, which can cause  a flash fire or an explosion. When adding cellulose ester to any flammable liquid, an inert gas atmosphere should be maintained within the vessel.  Minimize dust generation and accumulation.  In the United States of America, refer to NFPA® Pamphlet No. 654, "Prevention of Fire and Dust Explosions in the Chemical, Dye, Pharmaceutical, and Plastics Industries." General Storage

Storage:  Keep container closed.  Storage Stability and Temperature Tolerance

- EXPOSURE CONTROLS/PERSONAL PROTECTION

Country specific exposure limits have not been established or are not applicable unless listed below.

Exposure Limits

Ventilation

Ventilation: Good general ventilation (typically 10 air changes per hour) should be used. Ventilation rates should be matched to conditions. Supplementary local exhaust ventilation, closed systems, or respiratory protection may be needed in special circumstances such as poorly ventilated spaces, mechanical generation of dusts, heating, drying, etc.

Respiratory Protection

Respiratory Protection: If engineering controls do not maintain airborne concentrations to an acceptable level, an approved respirator must be worn. In the United States of America, if respirators are used, a program should be instituted to assure compliance with OSHA Standard 63 FR 1152, January 8, 1998. Respirator type: dust

Eye Protection

Eye Protection: It is a good industrial hygiene practice to minimize eye contact.

Hand protection

Skin Protection: It is a good industrial hygiene practice to minimize skin contact.Body Protection

Recommended DecontamRecommended Decontamination Facilities: eye bath, washing facilities

- PHYSICAL AND CHEMICAL PROPERTIES

Form

Physical Form: solid (powder)

Color

Color: white

Odor

Odor: slight, characteristic

Specific Gravity

Specific Gravity: 1.16 - 1.26

vapor pressure for other component

Softening or melting point

Melting Point: 127 - 240 °C

Solubility

Solubility in Water: negligible

pH

pH: not applicable

Flashpoint

Flash Point: not applicable, combustible solid

Thermal decomposition

Thermal Decomposition Temperature: Thermal stability not tested. Low stability hazard expected at normal operating temperatures.

Dust Explosion Class

Dust Explosion Class: St 2 - strong explosion

- STABILITY AND REACTIVITY

Stability

| Stability: | Stability Not fully evaluated.Materials containing similar structural groups are normally stable. Impact |
| --- | --- |

Incompatibility

| Incompatibility: | Material reacts with strong oxidizing agents. |
| --- | --- |

Hazard Decomposition Products

Hazardous Polymerization

| Hazardous Polymerization: | Will not occur. |
| --- | --- |

- TOXICOLOGICAL INFORMATION

Acute toxicity data, if available, are listed below. Additional toxicity data may be available on request.  Data not available unless listed below

Oral

| Oral LD-50:(rat) | >6,400 mg/kg(highest dose tested) |
| --- | --- |

DERMAL

| Dermal LD-50: ( guinea pig) | > 1,000 mg/kg (highest dose tested) |
| --- | --- |

skin irritation

| Skin Irritation (guinea pig) | very slight |
| --- | --- |

Sensitization

| (guinea pig: | none |
| --- | --- |

- ECOLOGICAL INFORMATION

Acute toxicity data, if available, are listed below. Additional toxicity data may be available on request.  Data not available unless listed below

Eco Summary

This material has not been tested for environmental effects.

It is a high molecular weight polymer with a very low water solubility.

As such, it is expected to have a low biochemical oxygen demand and to cause essentially no oxygen depletion in aquatic systems.

It is expected to have a low potential to affect aquatic organisms,

secondary waste treatment microorganisms, and the germination and early growth of plants.

It is expected to be nonbiodegradable and unlikely to bioconcentrate.

In a spill situation this material may be visually unpleasant; however, it is not expected to cause any adverse environmental effects.

- DISPOSAL CONSIDERATIONS

Discharge, treatment, or disposal may be subject to national, state, or local laws. Incinerate.

- TRANSPORT INFORMATION

MarinePollutantMarine pollutant components: none unless listed below

ICAO

ICAO Status: Class not regulatedsub class 1sub class 2

IMDG

IMDG Status: Class not regulatedsub class 1sub class 2

- REGULATORY INFORMATION

SARA 313: none, unless listed belowSARA 313

Carcinogenicity Classification (components present at 0.1% or more):  none, unless listed below

IARCIARC for components

ACGIH

ACGIH for components

NTP

NTP for components

OSHA

OSHA for components

TSCATSCA (US Toxic Substances Control Act):  This product is listed on the TSCA inventory. Any impurities present in this product are exempt from listing.

DSL & CEPADSL (Canadian Domestic Substances List) and CEPA (Canadian Environmental Protection Act):  This product is listed on the DSL. Any impurities present in this product are exempt from listing.

EINECSEINECS (European Inventory of Existing Commercial Chemical Substances):  All components of this product are listed on EINECS. Any polymer present in this product has regulatory clearance under Directives of the European Union.

AICS/NICNASAICS / NICNAS (Australian Inventory of Chemical Substances and National Industrial Chemicals Notification and Assessment Scheme):  This product is listed on AICS or otherwise complies with NICNAS.

MITIMITI (Japanese Handbook of Existing and New Chemical Substances):  This product is listed in the Handbook or has been approved in Japan by new substance notification.

KOREAECL (Korean Toxic Substances Control Act):  This product is listed on the Korean inventory or otherwise complies with the Korean Toxic Substances Control Act.

- OTHER INFORMATION

Visit our website at www.EASTMAN.com or call 1-800-EASTMAN.

The information contained herein is based on current knowledge and experience; no responsibility is accepted that the information is sufficient or correct in all cases. Users should consider these data only as a supplement to other information. Users should make independent determinations of suitability and completeness of information from all sources to assure proper use and disposal of these materials, the safety and health of employees and customers, and the protection of the environment.

Highlighted areas indicate new or changed information.